Hi @channel Some important information about the final project and how to prepare your graphs for node classification.

- Prepare your building as a 3D CellComplex for easy visualisation.

- For connectivity, add “Apertures”. Each aperture should have a dictionary key called “door_type” and its value should be a STRING and one of:

  - “passage” (will be mapped in code automatically to 0)

  - “door” (will be mapped in code automatically to 1)

  - “entrance_door” (will be mapped in code automatically to 2)

A passage is an opening but has no actual door. Still, model it as a regular aperture.

Door is any regular door between rooms or between a room and a corridor

Entrance Door is the front door of an apartment.

- For your rooms (cells), make sure they have a dictionary key called “room_type” and its value should be a STRING and one of:

  - “bedroom” (will be mapped in code automatically to 0)

  - “livingroom” (will be mapped in code automatically to 1)

  - “kitchen” (will be mapped in code automatically to 2)

  - “dining” (will be mapped in code automatically to 3)

  - “corridor” (will be mapped in code automatically to 4)

  - “stairs” (will be mapped in code automatically to 5)

  - “storeroom” (will be mapped in code automatically to 6)

  - “bathroom” (will be mapped in code automatically to 7)

  - “balcony” (will be mapped in code automatically to 8)

- Once you have your CellComplex with the cell values and the aperture values assigned, make sure you add the apertures to the CellComplex using Topology.AddApertures(your_cell_complex, your_list_of_apertures, exclusive=False, subTopologyType=”Face”, tolerance=0.001)

- Once that is done, create your Graph:

  - graph = Graph.ByTopology(your_cell_complex, direct=False, directApertures=True)

  - Check the vertices and the edges of the graph and make sure they have the correct values.

  - Use this script as a guide:

- from topologicpy.Face import Face

- from topologicpy.CellComplex import CellComplex

- from topologicpy.Topology import Topology

- from topologicpy.Graph import Graph

- from topologicpy.Dictionary import Dictionary

- cc = CellComplex.Prism(wSides=1, vSides=1)

- cells = Topology.Cells(cc)

- d = Dictionary.ByKeyValue("room_type", "livingroom")

- cells[0] = Topology.SetDictionary(cells[0], d)

- d = Dictionary.ByKeyValue("room_type", "bedroom")

- cells[1] = Topology.SetDictionary(cells[1], d)

- f = Face.Rectangle(width=0.5, length=0.5, direction=[1,0,0])

- d = Dictionary.ByKeyValue("door_type", "door")

- f = Topology.SetDictionary(f, d)

- cc = Topology.AddApertures(cc, [f], subTopologyType="Face")

- graph = Graph.ByTopology(cc, direct=False, directApertures=True)

- vertices = Graph.Vertices(graph)

- for v in vertices:

- d = Topology.Dictionary(v)

- print(Dictionary.Keys(d), Dictionary.Values(d))

- edges = Graph.Edges(graph)

- for e in edges:

- d = Topology.Dictionary(e)

- print(Dictionary.Keys(d), Dictionary.Values(d))

6. Once you have your graph ready, give it as input to S06-15B GML Prepare Graph for Node Classification.ipynb

7. Export your graph to CSV and check that everything is correct.

8. Load the graph and the pre-trained model (msd_node_classifier.pt) into S06-15C GML Predict MSD Graph.ipynb and run to see the results. If you have only one graph, at the last cell, choose graphs[0]